Stormwater Implementation Strategies

In the watershed areas covered by the Greater Trinity River Bacteria TMDL I-Plan Project, as in most urban areas, stormwater runoff is a major cause of water pollution. When rain falls on less developed areas, the water is absorbed and filtered by soil and plants. When rain falls on the roofs, streets, and parking lots of the Dallas-Fort Worth metropolitan area, however, the water cannot soak into the ground. Here, like most urban areas, stormwater is drained through engineered collection systems and discharged into nearby lakes and streams. The stormwater carries trash, heavy metals, other pollutants, and notably for this project, bacteria, from the urban landscape, degrading the quality of the receiving waters. Higher flows can also cause erosion and flooding in urban streams, damaging habitat, property, and infrastructure.

About the Regional Stormwater

Management Program

NCTCOG works with local governments and other stakeholders to develop and implement a regional strategy to address stormwater quality issues impacting the region. Created in 1999 by the Regional Stormwater Management Coordinating Committee (RSWMCC), the Regional Policy Position on Managing Urban Stormwater Quality provides guidance for the regional strategy, setting out the key elements for a cooperative and comprehensive regional approach to stormwater management. Among the goals of the Regional Program are to:

- Protect the health and welfare of citizens and the environment;

- Effectively address state and federal regulations;

- Share professional knowledge and experience; and

- Provide training to governmental staff and the development community.

The program is built upon a series of cooperative initiatives in the following areas:

- Public education;

- Control of construction site stormwater runoff;

- Management of stormwater impacts associated with post-construction;

- Illicit discharge detection and elimination; and

- Municipal pollution prevention.

- Regional cooperative monitoring

Bacteria sources, such as waste from pets, wildlife, and even humans, can be washed into storm drains and then discharged into local waterways. Because stormwater systems are designed to quickly and efficiently remove stormwater from developments, stormwater often bypasses the natural vegetative barriers that filter sheet flow over the land, thus, exacerbating bacteria loading. Infrastructure, such as pipes, inlets, culverts, interceptors, basins, reservoirs, outfalls, and channelized waterways, can also increase direct bacterial loading. The TMDLs for the project area indicate that stormwater from permitted MS4s is thought to be a significant source of bacteria loading (TCEQ 2011a and 2011b).

Effective stormwater management is often achieved from a management systems approach, as opposed to one that focuses on individual practices. That is, the pollutant control achievable from any given management system is viewed as the sum of the parts, taking into account the range of effectiveness associated with each single practice, the costs of each practice, and the resulting overall cost and effectiveness. Some individual practices may not be very effective alone but, in combination with others, may provide a key function in highly effective systems and, in the case of the Dallas-Fort Worth metropolitan area, reduce bacteria levels in area waterways.

Once high levels of bacteria are present in a water body, it is more difficult and expensive to restore it to a less impacted condition. The widespread use of BMPs for pollution prevention, illicit discharge detections, and elimination (IDDE), erosion and sediment control, and outreach and education are critical in meeting water quality goals for the Trinity River and its tributaries.

Existing requirements of MS4 permits address some important elements of bacteria loading in stormwater, offering an adaptive rather than prescriptive approach to bacteria reduction. Structural BMPs, such as modifications to stormwater outfalls that may reduce bacteria through aeration, treatment by sunlight, or physical removal of contaminants, have the potential to reduce bacteria loading into waterways. Because there is limited data regarding how well such BMPs might reduce bacteria loading, the Coordination Committee has identified the evaluation of the effectiveness of stormwater implementation activities as one of the top research priorities. Any research, particularly research relevant to the Greater Trinity area, should be reported and shared with Project stakeholders, so that stakeholders can devise appropriate strategies for integrating structural stormwater BMPs into their activities (see Implementation Strategy 8.0).

A map of MS4s in the project area is shown in Figure 9. A list of stormwater permits in the project area is provided in Tables 21, 22, 23, and 24.

## Implementation Strategy 2.0:  MS4 participation in Regional Stormwater Management Program

Local and state governments along with transportation entities with MS4 permits currently employ extensive and innovative stormwater programs, and many participate in the Regional Stormwater Management Program (RSWMP). The RSWMP already includes several programs relevant to bacteria loading and this I-Plan. The programs include Construction, Illicit Discharge, Monitoring, Pollution Prevention, and Public Education. Additionally, regionally developed initiatives and cooperative purchases are also part of the program. Because of the extensive involvement of the RSWMP in existing stormwater efforts, as well as its regional scope and contacts, partnering with the program and supporting the inclusion of bacteria-specific elements is the logical choice and takes advantage of existing knowledge and infrastructure. A list of RSWMP participants can be found in Table 25, while a summary of this implementation strategy can be found in Table 18.

### 2.0.1:  Request Regional Stormwater Management Coordinating Council include bacteria in RSWMP program efforts and materials

Given the broad scope of RSWMP programs and tools, the Coordination Committee requests the Regional Stormwater Management Coordinating Council (RSWMCC) direct their committees to review each program’s materials for inclusion of relevant information on bacteria load reduction.

#### 2.0.1.1:  IDDE program participation

An illicit discharge is defined as any discharge to the MS4 that is not composed entirely of stormwater (except for discharges allowed under a TPDES permit). Non-stormwater discharges can originate from direct connections to the storm drain system, from business or commercial establishments (illicit connections), or indirectly as improper surface discharges to the storm drain system.

Illicit plumbing connections may be intentional or may be unknown to a property owner and often are due to the connection of floor drains to the storm sewer system. As a result of these illicit connections, wastewater that should receive treatment from a WWTF directly enters storm drains and local surface waters and subsequently negatively impacts bacteria loading. Additional sources of illicit discharges may come from failing septic systems, illegal dumping practices, and the improper disposal of sewage from recreational practices such as boating or camping.

NCTCOG and the Coordination Committee encourage all MS4s within the Project area to participate in the RSWMP and continue and expand, where necessary, their programs for IDDE through participation in existing training and educational initiatives. Stakeholders also encourage the RSWMP’s IDDE Task Force to introduce or add bacteria-enhancing pollutant detection training and materials with examples from slaughter facilities, pet training/housing, farmers markets, sewage processors, zoos, etc.

#### 2.0.1.2:  Inclusion of bacteria load reduction in Pollution Prevention Peer-to-Peer program and evaluation of modified Peer-to-Peer program for five years

Peer-to-Peer is a program of the RSWMP’s Pollution Prevention (P2) Task Force. The program provides site visits to assess good housekeeping procedures in MS4s which can result in cost savings in production, materials, and disposal; increase public awareness of local water quality issues; and provide safer working conditions for city/county staff.

The Coordination Committee requests the RSWMCC direct the P2 Task Force to expand the existing Peer-to-Peer review program to include awareness about good housekeeping procedures that may help reduce bacteria loading. Additionally, the Committee requests the P2 Task Force continue the modified Peer-to-Peer program over a five-year permit term allowing for reevaluation of program effectiveness.

This strategy is being recommended to be moved to the Education and Outreach strategy scope. Task force meeting attendees expressed there was interest in the program and its effectiveness, but it is not able to be completed as outlined within the Regional Stormwater Management Program's Pollution Prevention (P2) Task Force.

Table 18. Implementation Strategy 2.0 Summary — MS4 participation in Regional Stormwater Management Program

| Targeted Source(s) | Stormwater |
| --- | --- |
| Estimated Potential Load Reduction | IS 2.0 – 2.0.1.2 may result in a 10% reduction over 25 years by contributing to the reduction of the stormwater bacteria load through education and cooperative efforts among various stakeholders |
| Technical and Financial Assistance Needed | Technical: participation in the RSWMP provides technical assistance for MS4s under several areas including construction, illicit discharge, monitoring, pollution prevention, and public education; some technical assistance may be necessary for the RSWMP to incorporate bacteria in their programs Financial: participation in the RSWMP is based on cost share and varies depending on MS4 size; inclusion of bacteria information is unlikely to exceed existing funding sources |
| Education Component | Outreach to non-participating MS4s regarding benefits of participation in the RSWMP Outreach to RSWMP's RSWMCC for inclusion of bacteria in their existing programs |
| Schedule of Implementation | Beginning immediately as appropriate Coordination Committee members, technical subcommittee members, and NCTCOG will conduct outreach to non-participants regarding benefits of RSWMP Beginning immediately as appropriate Coordination Committee and technical subcommittee members already involved in RSWMP and/or the RSWMCC will approach the RSWMCC regarding inclusion of bacteria in existing programs and materials |
| Interim, Measurable Milestone | Non-RSWMP MS4s approached RSWMCC approached for inclusion of bacteria in materials and programs |
| Progress Indicators | Number of RSWMP participants increases Bacteria-specific information included in RSWMP programs and materials |
| Monitoring Component | NCTCOG will collect data on RSWMP participation and programs and materials |
| Responsible Entity | Coordination Committee and technical subcommittee members with ties to RSWMP and/or RSWMCC will conduct outreach to non-participating MS4s and RSWMCC RSWMCC will consider inclusion of bacteria-specific information in RSWMP outreach materials and programs NCTCOG will assist the Coordination Committee and Stormwater technical subcommittee with outreach and will present participation data and material and program updates annually to the Coordination Committee and Stormwater technical subcommittee |

## Implementation Strategy 2.1:  Local Supplemental Environmental Projects

At the state level, the TCEQ defines supplemental environmental projects (SEPs) as, “[A] project that prevents pollution, reduces the amount of pollution reaching the environment, enhances the quality of the environment, or contributes to public awareness of environmental matters.” A respondent in an enforcement action may negotiate an agreement to perform a SEP in return for an offset of the administrative penalty. The proposal to include a particular SEP in an agreed order will be presented to the Commission or Executive Director for consideration and final approval. Potential SEPs include such diverse projects as cleanups of abandoned tire sites or illegal dump sites, community collections of household hazardous waste, and pollution prevention projects that exceed regulatory requirements. SEPs that have a direct benefit allow a respondent to offset one dollar of its penalty for every dollar spent on the SEP (TCEQ, 2012a).”

Detailed in Table 19, the purpose of Implementation Strategy 2.1 is to bring the idea of SEPs to the local level — outside of the scope of the state and solely the purview of the individual local jurisdiction. Local SEPs are intended for watershed improvements and other environmentally beneficial projects that a respondent agrees to undertake in settlement of an enforcement action, but which the respondent is not otherwise legally required to perform, and for which he/she does not receive any other benefit. The local SEPs can be negotiated through the regulatory enforcement process with the city or other regulated MS4s with enforcement capabilities.

The Coordination Committee encourages local municipalities to adopt or continue using local SEPs — separate, but not to the exclusion of the state SEP program — in addition to fines, as part of escalating enforcement programs for unfunded local stormwater projects to reduce bacteria loading. As such, a goal of 75 percent of large municipal MS4s within bacteria-impaired watersheds will have local SEPs as part of stormwater enforcement by 2028 and 25 percent of small municipal MS4s will have such a program by 2033.

Add additional information about the differences between the compliance sep vs 3rd party administrators

The description of this strategy is being revised to include additional information about the differences between compliance SEPs vs 3rd party administrators.

The Schedule of Implementation goal is being recommended to be used as a tracking tool rather than a program requirement. There was also interest in more educational resources/components surrounding SEPs overall.

The timelines for the Schedule of Implementation and the Interim, Measurable Milestone are being recommended to have more feasibility in the dates for large and small municipal MS4s to obtain SEP programs.

Table 19. Implementation Strategy 2.1 Summary — Local Supplemental Environmental Projects

| Targeted Source(s) | Stormwater |
| --- | --- |
| Estimated Potential Load Reduction | IS 2.1 may result in a 4% reduction over 25 years by providing an additional source of funds that can be used for projects that will reduce bacterial loads. Use of local SEPs may also better engage violators in the process of improving water quality locally |
| Technical and Financial Assistance Needed | Technical: technical assistance may be necessary for entities to implement their own local SEP program Financial: existing funding as appropriate |
| Education Component | Information will be made available for local SEP implementation |
| Schedule of Implementation | 75% of large municipal MS4s will have local SEP programs in place by 2028 25% of small municipal MS4s will have local SEP programs in place by 2033 More of a tracking tool rather than program acquisition/requirement Educational component/resources? Modification of verbiage for feasibility. |
| Interim, Measurable Milestone | By 2023, 50% of large municipal MS4s will have local SEP programs in place By 2028, 15% of small municipal MS4s will have local SEP programs in place |
| Progress Indicators | Number of municipal MS4s with local SEP programs |
| Monitoring Component | NCTCOG will collect data on municipal MS4 local SEP programs |
| Responsible Entity | Municipal MS4s will adopt local SEP programs as feasible NCTCOG will compile information on SEP programs for an annual report to Coordination Committee and Stormwater technical subcommittee |

## Implementation Strategy 2.2:  Land use, business, and regulatory review

Analyses by the Project’s technical review subcommittee members revealed a potential gap in many existing stormwater codes and regulations with respect to addressing discharges with the potential to carry bacteria. As currently written, many rules, including the base stormwater discharge permits, focus on chemical or physical constituents, such as toxic chemicals or sediment, but may not completely address bacterial sources or discharges. Examples of facilities that may pose a risk for bacterial discharge include but are not limited to:  slaughterhouses and meat-processing facilities, stables and pet-boarding facilities, sewage processors, produce packing facilities, and farmer’s markets. Implementation strategies for land use and business evaluation are summarized in Table 20.

### 2.2.1:  Business risk evaluation and enforcement

Municipalities will review their respective codes and ordinances and, as feasible, revise as necessary to address the discharge of bacteria, nutrients, and other substances that could contribute to bacterial growth in the environment.

### 2.2.2:  Request to TCEQ for Industrial Stormwater Multi-Sector General Permit classification review and benchmark bacteria monitoring

TCEQ is encouraged to review, and as necessary amend the TPDES No. TXR050000, Multi-Sector General Permit (MSGP) to require facilities located in bacteria-impaired watersheds with operations having the potential to discharge bacteria, (such as the current Sector U, Food and Kindred Products Facilities), to perform benchmark sampling for bacteria.

- The Estimated Potential Load Reduction goal is being recommended to revise "IS 2.2 – 2.2.2 may result in a 2 the 2% reduction in bacteria loading as problems are identified and corrected over 25 years" to IS 2.2 – 2.2.2 may result in reduction of bacterial loading ss problems are identified and corrected over 25 years”

- The Responsible Entity goal is being recommended to shift from "presenting annually to the TMDL Coordination Committee and Stormwater technical subcommittee" to  "presenting every 5 years to the TMDL Coordination Committee and Stormwater technical subcommittee25".

- The adjusted scope of this strategy is being recommended to be more feasible or be eliminated due to lack of necessity and capacity of responsible entities.

Table 20. Implementation Strategy 2.2 Summary — Land use, business, and regulatory review

| Targeted Source(s) | Businesses/facilities at risk for bacterial discharge |
| --- | --- |
| Estimated Potential Load Reduction | IS 2.2 – 2.2.2 may result in a 2% reduction in bacteria loading as problems are identified and corrected over 25 years “May result in reduction of bacterial loading” |
| Technical and Financial Assistance Needed | Technical: technical assistance may be necessary for MS4s to develop and study their own land use and evaluate businesses with potential to discharge bacteria Financial: loans, grant funding and existing funding as appropriate |
| Education Component | Outreach to MS4s concerning land use and business evaluation may be necessary Educational efforts by MS4s regarding operations and land use to businesses with potential to discharge bacteria Outreach to impacted businesses should TCEQ amend MSGP requirements |
| Schedule of Implementation | As resources are available, the implementation of this activity will begin immediately and will continue for the entire implementation process |
| Interim, Measurable Milestone | Number of local evaluations completed Number of site visits to businesses with potential to discharge bacteria |
| Progress Indicators | Number of reported program expansion and/or modifications to address high risk businesses Changes to MSGP requirements |
| Monitoring Component | NCTCOG will collect data on local efforts and any changes to the TCEQ MSGP |
| Responsible Entity | MS4s will evaluate local land use and businesses for potential for bacteria discharges NCTCOG will coordinate dialogue between MS4s, stakeholders and TCEQ to identify potential modifications to the MSGP that will aid in addressing bacteria as a pollutant and benchmark bacteria monitoring NCTCOG will compile data collected on local efforts and any changes to the TCEQ MSGP and present it annually to Coordination Committee and Stormwater technical subcommittee - Replace with something more feasible - Present every 5 years rather than annually |

Additional stakeholder feedback:

Perform analysis of load reductions – we identified what was needed, but how close are we to meeting those goals?

Table 21. MS4 Permittees by AU for 0805 and 0822 Segments

| AU | MS4 Permittees | River System | Segment | TPDES Permit Number |
| --- | --- | --- | --- | --- |
| 0805_03 | City of Dallas | Upper Trinity | Fivemile Creek upstream to the confluence of Cedar Creek | WQ0004396 |
| 0805_03 | TxDOT – Dallas | Upper Trinity | Fivemile Creek upstream to the confluence of Cedar Creek | WQ0004521 |
| 0805_03 | North Texas Tollway Authority | Upper Trinity | Fivemile Creek upstream to the confluence of Cedar Creek | WQ0004400 |
| 0805_03 | Dallas Area Rapid Transit | Upper Trinity | Fivemile Creek upstream to the confluence of Cedar Creek | TXR040000 |
| 0805_04 | City of Dallas | Upper Trinity | Confluence of Cedar Creek upstream to confluence of Elm Fork Trinity River | WQ0004396 |
| 0805_04 | City of Irving and co-permittees: Dallas Co. Flood Control District #1, Dallas County Utility & Reclamation District, Irving Flood Control District Sections I & III | Upper Trinity | Confluence of Cedar Creek upstream to confluence of Elm Fork Trinity River | WQ0004691 |
| 0805_04 | TxDOT – Dallas | Upper Trinity | Confluence of Cedar Creek upstream to confluence of Elm Fork Trinity River | WQ0004521 |
| 0805_04 | North Texas Tollway Authority | Upper Trinity | Confluence of Cedar Creek upstream to confluence of Elm Fork Trinity River | WQ0004400 |
| 0805_04 | City of University Park | Upper Trinity | Confluence of Cedar Creek upstream to confluence of Elm Fork Trinity River | TXR040000 |
| 0805_04 | Town of Highland Park | Upper Trinity | Confluence of Cedar Creek upstream to confluence of Elm Fork Trinity River | TXR040000 |
| 0805_04 | City of Cockrell Hill | Upper Trinity | Confluence of Cedar Creek upstream to confluence of Elm Fork Trinity River | TXR040000 |
| 0805_04 | Dallas Area Rapid Transit | Upper Trinity | Confluence of Cedar Creek upstream to confluence of Elm Fork Trinity River | TXR040000 |
| 0805_04 | Buckley Oil Company b | Upper Trinity | Confluence of Cedar Creek upstream to confluence of Elm Fork Trinity River | WQ0004663 |
| 0822A | City of Irving and co-permittees: Dallas Co. Flood Control District #1, Dallas County Utility & Reclamation District, Irving Flood Control District Sections I & III | Cotton Wood Branch | 2.5 mile stretch of Cottonwood Branch running upstream from confluence with Hackberry Creek | WQ0004691 |
| 0822A | North Texas Tollway Authority | Cotton Wood Branch | 2.5 mile stretch of Cottonwood Branch running upstream from confluence with Hackberry Creek | WQ0004400 |
| 0822A | DFW International Airport a | Cotton Wood Branch | 2.5 mile stretch of Cottonwood Branch running upstream from confluence with Hackberry Creek | TXR040000 |
| 0822B | City of Irving and co-permittees: Dallas Co. Flood Control District #1, Dallas County Utility & Reclamation District, Irving Flood Control District Sections I & III | Grapevine Creek | Entire water body | WQ0004691 |
| 0822B | City of Dallas | Grapevine Creek | Entire water body | WQ0004396 |
| 0822B | TxDOT– Dallas | Grapevine Creek | Entire water body | WQ0004521 |
| 0822B | City of Coppell | Grapevine Creek | Entire water body | TXR040000 |
| 0822B | DFW International Airport a | Grapevine Creek | Entire water body | TXR040000 |

a Includes five outfalls covered under an individual industrial stormwater permit (WQ0001441).

b Individual industrial stormwater permit included as part of the MS4 allocation.

Table 22. MS4 Permittees by AU for 0841 Segments

| AU | MS4 Permittees | River System | Segment | TPDES Permit Number |
| --- | --- | --- | --- | --- |
| 0841_01 | City of Irving and co-permittees: Dallas Co. Flood Control District #1, Dallas County Utility & Reclamation District, Irving Flood Control District Sections I & III | Lower West Fork Trinity River | Confluence of the Elm Fork Trinity River to the confluence with Johnson Creek | WQ0004691 |
| 0841_01 | City of Arlington and co-permittees: University of Texas at Arlington and TxDOT-Fort Worth | Lower West Fork Trinity River | Confluence of the Elm Fork Trinity River to the confluence with Johnson Creek | WQ0004635 |
| 0841_01 | City of Dallas | Lower West Fork Trinity River | Confluence of the Elm Fork Trinity River to the confluence with Johnson Creek | WQ0004396 |
| 0841_01 | City of Fort Worth and co-permittees: Tarrant Regional Water District, and TxDOT-Fort Worth | Lower West Fork Trinity River | Confluence of the Elm Fork Trinity River to the confluence with Johnson Creek | WQ0004350 |
| 0841_01 | North Texas Tollway Authority | Lower West Fork Trinity River | Confluence of the Elm Fork Trinity River to the confluence with Johnson Creek | WQ0004400 |
| 0841_01 | TxDOT – Dallas | Lower West Fork Trinity River | Confluence of the Elm Fork Trinity River to the confluence with Johnson Creek | WQ0004521 |
| 0841_01 | City of Grand Prairie | Lower West Fork Trinity River | Confluence of the Elm Fork Trinity River to the confluence with Johnson Creek | TXR040000 |
| 0841_01 | City of Keller | Lower West Fork Trinity River | Confluence of the Elm Fork Trinity River to the confluence with Johnson Creek | TXR040000 |
| 0841_01 | City of Colleyville | Lower West Fork Trinity River | Confluence of the Elm Fork Trinity River to the confluence with Johnson Creek | TXR040000 |
| 0841_01 | City of Southlake | Lower West Fork Trinity River | Confluence of the Elm Fork Trinity River to the confluence with Johnson Creek | TXR040000 |
| 0841_01 | City of Grapevine | Lower West Fork Trinity River | Confluence of the Elm Fork Trinity River to the confluence with Johnson Creek | TXR040000 |
| 0841_01 | City of Euless | Lower West Fork Trinity River | Confluence of the Elm Fork Trinity River to the confluence with Johnson Creek | TXR040000 |
| 0841_01 | City of North Richland Hills | Lower West Fork Trinity River | Confluence of the Elm Fork Trinity River to the confluence with Johnson Creek | TXR040000 |
| 0841_01 | City of Bedford | Lower West Fork Trinity River | Confluence of the Elm Fork Trinity River to the confluence with Johnson Creek | TXR040000 |
| 0841_01 | City of Hurst | Lower West Fork Trinity River | Confluence of the Elm Fork Trinity River to the confluence with Johnson Creek | TXR040000 |
| 0841_01 | Tarrant County | Lower West Fork Trinity River | Confluence of the Elm Fork Trinity River to the confluence with Johnson Creek | TXR040000 |
| 0841_01 | Dallas County | Lower West Fork Trinity River | Confluence of the Elm Fork Trinity River to the confluence with Johnson Creek | TXR040000 |
| 0841_01 | TxDOT – Fort Worth | Lower West Fork Trinity River | Confluence of the Elm Fork Trinity River to the confluence with Johnson Creek | TXR040000 |
| 0841_01 | DFW International Airport a | Lower West Fork Trinity River | Confluence of the Elm Fork Trinity River to the confluence with Johnson Creek | TXR040000 |
| 0841_01 | Dallas Area Rapid Transit | Lower West Fork Trinity River | Confluence of the Elm Fork Trinity River to the confluence with Johnson Creek | TXR040000 |
| 0841_01 | North Texas Tollway Authority | Lower West Fork Trinity River | Confluence of the Elm Fork Trinity River to the confluence with Johnson Creek | TXR040000 |
| 0841_01 | Dallas Co. Flood Control Dist. No. 1 | Lower West Fork Trinity River | Confluence of the Elm Fork Trinity River to the confluence with Johnson Creek | TXR040000 |
| 0841_01 | Extex LaPorte LP b | Lower West Fork Trinity River | Confluence of the Elm Fork Trinity River to the confluence with Johnson Creek | WQ0001250 |

a Includes five outfalls covered under an individual industrial stormwater permit (WQ0001441).

b Individual industrial stormwater permit included as part of the MS4 allocation.

| AU | MS4 Permittees | River System | Segment | TPDES Permit Number |
| --- | --- | --- | --- | --- |
| 0841_02 | City of Arlington and co-permittees: University of Texas at Arlington and TxDOT-Fort Worth | Lower West Fork Trinity River | From the confluence with Johnson Creek upstream to the confluence of Village Creek. | WQ0004635 |
| 0841_02 | City of Fort Worth and co-permittees: Tarrant Regional Water District, and TxDOT-Fort Worth | Lower West Fork Trinity River | From the confluence with Johnson Creek upstream to the confluence of Village Creek. | WQ0004350 |
| 0841_02 | TxDOT – Dallas | Lower West Fork Trinity River | From the confluence with Johnson Creek upstream to the confluence of Village Creek. | WQ0004521 |
| 0841_02 | City of North Richland Hills | Lower West Fork Trinity River | From the confluence with Johnson Creek upstream to the confluence of Village Creek. | TXR040000 |
| 0841_02 | City of Hurst | Lower West Fork Trinity River | From the confluence with Johnson Creek upstream to the confluence of Village Creek. | TXR040000 |
| 0841_02 | City of Bedford | Lower West Fork Trinity River | From the confluence with Johnson Creek upstream to the confluence of Village Creek. | TXR040000 |
| 0841_02 | City of Grand Prairie | Lower West Fork Trinity River | From the confluence with Johnson Creek upstream to the confluence of Village Creek. | TXR040000 |
| 0841_02 | City of Euless | Lower West Fork Trinity River | From the confluence with Johnson Creek upstream to the confluence of Village Creek. | TXR040000 |
| 0841_02 | City of Dalworthington Gardens | Lower West Fork Trinity River | From the confluence with Johnson Creek upstream to the confluence of Village Creek. | TXR040000 |
| 0841_02 | Town of Pantego | Lower West Fork Trinity River | From the confluence with Johnson Creek upstream to the confluence of Village Creek. | TXR040000 |
| 0841_02 | City of Kennedale | Lower West Fork Trinity River | From the confluence with Johnson Creek upstream to the confluence of Village Creek. | TXR040000 |
| 0841_02 | City of Colleyville | Lower West Fork Trinity River | From the confluence with Johnson Creek upstream to the confluence of Village Creek. | TXR040000 |
| AU | MS4 Permittees | River System | Segment | TPDES Permit Number |
| 0841_02 | Tarrant County | Lower West Fork Trinity River | From the confluence with Johnson Creek upstream to the confluence of Village Creek. | TXR040000 |
| 0841_02 | Tarrant County College NE | Lower West Fork Trinity River | From the confluence with Johnson Creek upstream to the confluence of Village Creek. | TXR040000 |
| 0841_02 | TxDOT-Fort Worth | Lower West Fork Trinity River | From the confluence with Johnson Creek upstream to the confluence of Village Creek. | TXR040000 |
| 0841_02 | Dallas Area Rapid Transit | Lower West Fork Trinity River | From the confluence with Johnson Creek upstream to the confluence of Village Creek. | TXR040000 |

Table 23. TPDES and NPDES MS4 Permits associated with Segments 0841F, 0841K, 0841N, and 0841V

| AU | MS4 Permittees | River System | Segment | TDPES Permit Number |
| --- | --- | --- | --- | --- |
| 0841K_01 | City of Arlington | Fish Creek | From the confluence with Mountain Creek Reservoir in Grand Prairie, Dallas Co, to the upper end of the creek in Arlington, Tarrant Co. | TXS000301 |
| 0841V_01 | City of Dallas | Crockett Branch | From the confluence with Cottonwood Creek to the upper end of the creek. | TXS000701 |
| 0841F_01 | City of Grand Prairie | Kirby Creek | From the confluence with Fish Creek in Grand Prairie, Dallas Co. to just upstream of Great Southwest Parkway in Arlington, Tarrant Co. | TXR040065 |
| 0841N_01 | Dallas County | Cottonwood Creek | 6.5 mile stretch of Cottonwood Creek running upstream from approx. 1 mi. upstream of Mountain Creek Reservoir in Dallas Co. to SH 360 in, Tarrant Co. | TXR040120 |
| 0841K_01 | Tarrant County | Fish Creek | From the confluence with Mountain Creek Reservoir in Grand Prairie, Dallas Co, to the upper end of the creek in Arlington, Tarrant Co. | TXR040052 |

Table 24. TPDES and NPDES MS4 Permits associated with Segment 0806E

| AU | MS4 Permittees | River System | Segment | NDPES Permit Number |
| --- | --- | --- | --- | --- |
| 0806E_01 | City of Fort Worth, Tarrant Regional Water District | Sycamore Creek | A 5 mile stretch of Sycamore Creek running upstream from the confluence with the West Fork Trinity River to the confluence with Echo Lake Tributary in Fort Worth. | TXS000901 |
| 0806E_01 | Texas Department of Transportation | Sycamore Creek | A 5 mile stretch of Sycamore Creek running upstream from the confluence with the West Fork Trinity River to the confluence with Echo Lake Tributary in Fort Worth. | TXS002101 |
| 0806E_01 | Town of Edgecliff Village | Sycamore Creek | A 5 mile stretch of Sycamore Creek running upstream from the confluence with the West Fork Trinity River to the confluence with Echo Lake Tributary in Fort Worth. | TXR040595 |
| 0806E_01 | Tarrant County | Sycamore Creek | A 5 mile stretch of Sycamore Creek running upstream from the confluence with the West Fork Trinity River to the confluence with Echo Lake Tributary in Fort Worth. | TXR040052 |

Table 25. RSWMP Participation in Project Area as of FY2019

| Cost Share Regional Stormwater Management Program (RSWMP) Participants | Non-Participants |
| --- | --- |
| City of Arlington | City of Cockrell Hill |
| City of Bedford | City of Dalworthington Gardens |
| City of Colleyville | City of Haslet |
| City of Coppell | City of Keller |
| City of Dallas | North Texas Tollway Authority |
| City of Euless | TxDOT Dallas District |
| City of Fort Worth | Town of Pantego |
| City of Grand Prairie |  |
| City of Grapevine |  |
| City of Hurst |  |
| City of Irving |  |
| City of Kennedale |  |
| City of North Richland Hills |  |
| City of Southlake |  |
| City of University Park |  |
| Dallas Area Rapid Transit |  |
| DFW Airport |  |
| Dallas County |  |
| Tarrant County |  |
| TxDOT Fort Worth District |  |
| Town of Highland Park |  |

Figure 9. MS4s in Project Area